VD201 Vibration Sensor

Summary

This is one NC vibration sensor, can change the vibration signal into switch signal. Three sensitivities adjustable apply to different environment.

Technical parameter

Contact point current max: 20mA

Contact point current min: 10mA

Sensitivity: high, medium, low

Carried standard: GB/10408-2008

Installation

Drill mounting holes in the area where need to be detected, refer to Chart one.

Fix the sensor bracket by screws.

The sensor should be parallel with the installation area.

When can’t fix by screws, 3M sponge adhesive can be use to fix the sensor.

Notice: for reducing false alarm, please avoid install the vibration sensor in the area prone to vibration.

Sensitivity adjustment

Open the plastic house of the sensor, you may see H, M, L and L four letters. H means high sensitivity, M means medium sensitivity, L means low sensitivity. Please choose the proper sensitivity when installing. For example, high sensitivity is selected, the letter L should be always upwards.

The guarantee period of this product is three years, man-made and force majors failures are not within this scope.